Residential Design

Guidelines

Contents

1.0

INTRODUCTION

1

7.0

COTTAGE LOTS

27

27

27

27

28

28

28

28

28

7.1

7.2

7.3

7.4

7.5

7.6

7.7

7.8

Minimum Setbacks Open Space Outdoor Living Area Parking

Stores & Outbuildings Fences

Letterbox Location

Duplex Lots

2.0

ARCHITECTURAL CHARACTER

3

3.0

RESTRICTIVE COVENANTS AND DETAILED AREA PLANS

5

3.1

3.2

3.3

Restrictive Covenants Detailed Area Plans

Other Relevant Documents

5

5

5

4.0

SUSTAINABLE DEVELOPMENT

7

9

4.1

Passive Environmental Home Design

Energy Use Water Use Garden Design

Waste And Recycling

Community & Social Considerations

“Quiet House” Design

8.0

TRADITIONAL LOTS

29

29

29

29

30

30

30

8.1

8.2

8.3

8.4

8.5

8.6

Minimum Setbacks Open Space Outdoor Living Area Stores

Fences

Bushfire Design Considerations

4.2

4.3

4.4

4.5

4.6

12

13

13

14

16

4.7

18

9.0

TOWNHOUSE LOTS

31

31

31

31

31

31

31

31

32

32

32

9.1

9.2

9.3

9.4

9.5

9.6

9.7

9.8

9.9

Building Form Minimum Setbacks

Site Levels And Retaining Walls Open Space

Outdoor Living Area Parking

Stores & Outbuildings Fences

Letterbox Location

5.0

COMMON CONDITIONS

19

19

22

23

23

23

24

5.1

5.2

5.3

5.4

5.5

5.6

Building Form Building Materials Street Presentation Services

Fencing Generally

Existing Mature Trees

6.0

PARKLAND LOTS

25

25

25

25

26

26

26

26

6.1

6.2

6.3

6.4

6.5

6.6

6.7

Minimum Setbacks Open Space Outdoor Living Area Street Outlook

Stores & Outbuildings Fences

Duplex Lots

9.10 Duplex Lots

  - HOMESTEAD LOTS

  - Minimum Setback

  - Tree  Preservation Area

  - Construction Materials

  - Fences

  - Bushfire Design Consideration

33

33

33

33

33

34

1

Introduction

The Village at Wellard has been designed to  create a compact transit orientated neighbourhood centred on the Wellard transit station. The residential subdivision features a mix of lot sizes and configurations aimed at optimising the range

of residential development opportunities.

These guidelines have been prepared to assist purchasers in developing their properties to a standard which will maximise the value of their investment whilst enhancing the quality of the

estate as a whole.

In assessing design submissions for compliance with the applicable covenants, adherence to the principles of these guidelines will also be taken into consideration. Residential lots have been categorised in accordance with lot sizes and the

natural amenity they are afforded as follows:

1 | The Village at Wellard Residential Design Guidelines

The Village at Wellard Residential Design Guidelines | 2

PARKLAND LOTS

Lots fronting onto Public Open Space.

Approx area 330 – 440m² Approx frontage 12 – 15m

COTTAGE LOTS

Lots served by a rear laneway for vehicular access and garaging, resulting in a pedestrian friendly front street.

Approx area 286 – 440m² Approx frontage 10 – 14m.

TRADITIONAL LOTS

Lots generally with a single street frontage and vehicular access via the front.

Approx area 450 – 800m² Approx frontage 15 – 20m.

TOWNHOUSE LOTS

Lots close to the village centre served by rear lane way for vehicle access and garaging.

Approx area 220 – 330m² Approx frontage 8 – 11m.

HOMESTEAD LOTS

Large lifestyle lots adjacent to existing Special Residential development.

Approx area 1500 – 2100m² Approx frontage 27 – 33m.

2

Architectural Character

The architectural character is to be of a contemporary

nature with street elevations articulated to feature clearly defined elements. External wall finishes should feature:

The aim is to encourage residential development which reflects a contemporary Australian urbanism rather than historical or vernacular styles such as “Federation”,

“Tuscan,” etc.

- a primary material such as painted render, face brickwork, stonework or rammed earth

complemented by minor elements including painted or clear finished weatherboards, corrugated metal cladding, painted fibre cement panel cladding,

accent colours, etc.

INDICATIVE OF ARCHITECTURAL CHARACTER

3 | The Village at Wellard Residential Design Guidelines

3 Restrictive Covenants & Detailed Area Plans

3.1 RESTRICTIVE COVENANTS

3.3 OTHER RELEVANT DOCUMENTS

The lots are encumbered by restrictive covenants which are intended to benefit all property owners by ensuring minimum standards of development are met and the

amenity of all properties is optimised.

The design guidelines are to be read in conjunction with  the applicable Detailed Area Plans together with any other relevant and current statutory or contractual documents

including:

•

Building Code of Australia (BCA)

3.2 DETAILED AREA PLANS

•

Residential Design Codes of WA (R-Codes)

Detailed Area Plans are prepared for the lots to illustrate such elements as building envelopes, recommended outdoor living or courtyard locations, garage locations, R-Code variations, landmark building element locations,

duplex and group dwelling site locations, etc.

- Town of Kwinana Town Planning Scheme No 2 (TPS)

- Sales contract and appendices

5 | The Village at Wellard Residential Design Guidelines

The Village at Wellard Residential Design Guidelines | 6

4

Sustainable Development

The Wellard joint venture partners are committed to the promotion of a sustainable development. The Village at Wellard is a transit oriented subdivision featuring a variety of housing lot typologies including a mixed use village centre within a walkable neighbourhood which makes better use of infrastructure services and contributes to

energy efficient design.

The Sustainable Home

A sustainable home is one which contributes to the natural, social and economic environments by achieving certain performance levels in these areas whilst mitigating some of the negative impacts brought about by the development process. Minimum built form statutory performance criteria are being introduced and refined on an ongoing basis by the Commonwealth and State Governments in an effort

to achieve this goal.

Sustainability initiatives aim to respond to the needs of today’s and future generations by addressing a range of environmental, social and economic issues. In order to contribute to this policy, homes constructed within the estate should satisfy a range of socially responsible

sustainability criteria.

In the areas of energy and water efficiency, waste minimisation and recycling, buyers are recommended to

refer to the GreenSmart website www.greensmart.com.au

- The reduction of energy use from non renewable resources.

- Reduced waste from the building process resulting in economic efficiency and less waste going to landfill;

- The creation of a healthy and safe living environment which can cater for the changing requirements of an aging population.

- The creation of interesting built form streetscapes reflecting the rich mix of social diversity contained within;

7 | The Village at Wellard Residential Design Guidelines

- The creation of a safe and secure urban environment through the application of design techniques such as the maximisation of passive surveillance opportunities to limit opportunities for antisocial behaviour.

- Contributing to community values by fostering easy social interaction and passive surveillance opportunities;

- Reduced demand for non renewable natural resources, such as native hardwoods or rainforest timber;

- Reduced demand for scheme water;

KEY SUSTAINABILITY AIMS INCLUDE:

Guiding text aimed at contributing to sustainable building

practice is as follows:

- The scale, bulk and placement of the home on the site shall minimise the overshadowing effect on adjacent properties to within limits permitted in the

R Codes, thus allowing neighbours a suitable level

Guiding Text:

CROSS VENTILATION

•

Concrete floors and masonry walls create mass which can absorb and store heat from the sun in winter and

re-radiate it into the living area.

4.1 PASSIVE ENVIRONMENTAL HOME DESIGN

of solar access.

4.1.1 THE HOME IS ORIENTATED TO FACILITATE

NORTHERLY SOLAR ACCESS TO LIVING AREAS.

•

Thermal mass is best located in living areas which are

north facing.

•

When planting trees to solar accessible spaces, it is wise to strategically locate deciduous species to provide summer shade whilst permitting winter

solar penetration.

Good orientation facilitates energy efficiency by reducing the requirement to heat and cool the home. For parkland/ cottage/townhouse lots nominated zero side setbacks are permitted in order to better facilitate the provision of a solar

accessible courtyard along the opposite side boundary.

•

Hard surfaces such as tiles allow the sun to heat the slab more readily than barrier materials like timber/

carpet or cork.

•

The extent of glazing to the east and west sides of homes should be minimised. Where openings are provided to take advantage of westerly or easterly views, these should have protection such as solar

shutters, blinds or screening devices.

•

Darker coloured tiles will absorb heat more efficiently

than light tiles.

Guiding Text:

•

In summer the thermal mass should be shaded from

direct sunlight by the use of eaves, solar pergolas, awnings, blinds, etc.

•

Daytime living areas should be located such that major openings face north to allow greater winter solar penetration when the sun angle is lower. In summer when the sun travels overhead, openings should be shaded by eaves overhangs or awnings. The ideal orientation for living areas is within 15°W to 20°E of

true north.

4.1.2 THE DESIGN PERMITS GOOD CROSS

VENTILATION

THERMAL MASS

Good cross ventilation can reduce the need to use air

conditioning in summer and contribute to a healthy home.

Guiding Text:

•

North facing walls and windows requiring winter solar access should be set well back from large obstructions. Where overshadowing from neighbouring homes to the north is anticipated, areas requiring solar access should be set back from the neighbour sufficiently to allow

winter solar penetration.

4.1.3 THE HOME IS ZONED TO MAXIMISE THE

EFFICIENCY OF HEATING AND COOLING.

•

Habitable rooms shall have external openings located to facilitate through ventilation as required by the BCA. The south westerly sea breezes and night

time easterlies can help cool the home in summer  if windows are appropriately placed to permit

cross ventilation.

Large unzoned open plan homes will be more expensive to heat and cool, while air leakage through unsealed openings

can also add considerably to heating and cooling costs.

Guiding Text:

•

Outdoor living or courtyard areas should be located to the north to maximise solar access and to also facilitate

unobstructed solar penetration to indoor living areas.

•

Ceiling mounted sweep fans can be used to assist

with airflow in the room.

•

The home should be split into compartments to maximise the efficiency of heating and cooling. Ground and upper floors and different living areas should be capable of being closed off from one

another to create easily heated or cooled zones.

•

When selecting or designing a home, consider a plan which will have living areas facing north or a design that can be adjusted to incorporate passive solar

design principles.

•

Building products, furniture, paint, cleaning products and consumer goods can emit Volatile Organic Compounds (VOCs). The level of these chemicals which evaporate into the atmosphere at room temperature, together with pollens and dust can be

significantly reduced in homes with good ventilation

•

Chimneys should be fitted with dampers to prevent warm air escaping from the house when the fireplace

is not in use.

•

Each home should have a minimum of one (1) internal living area with a major opening located to facilitate

northerly solar access.

characteristics.

•

Doors and windows should be well sealed to prevent

warmth escaping or draughts affecting heated rooms.

PASSIVE ENVIRONMENTAL HOME DESIGN

4.1.4 THERMAL MASS HAS BEEN INCORPORATED

INTO THE HOME

Thermal mass describes the home’s ability to absorb, store and re-radiate heat. When effectively incorporated into the home this can help reduce fluctuations in the internal air temperature making for a more comfortable environment

which is less expensive to heat.

9 | The Village at Wellard Residential Design Guidelines

The Village at Wellard Residential Design Guidelines | 10

Parkland Lot	Cottage Lot	Traditional Lot

Layout needs to allow for ease of cross ventilation

4.1.5 THE HOME HAS BEEN PROPERLY INSULATED

4.1.6 OPEN SPACE AND OUTDOOR LIVING AREAS

4.2 ENERGY USE

4.2.2 MECHANICAL HEATING AND COOLING HAS

BEEN MINIMISED

Roof and ceiling insulation can contribute to considerable

savings on heating and cooling costs with additional savings possible by the addition of wall insulation.

Usable open space is an important consideration in the design of any home. Perth’s temperate climate enables outdoor living areas to be utilised for much of the year and accordingly an extent of open space is required to be provided as part of the amenity of a dwelling. In particular a

minimum outdoor living area is required.

4.2.1 A HOT WATER SYSTEM WHICH COMPLIES

WITH THE REQUIREMENTS OF ‘5 STAR PLUS’ SHALL BE INSTALLED.

•

Ceiling fans should be installed in the bedrooms.

•

Where an air conditioner or evaporative cooling

system is installed it should have a minimum

Guiding Text:

Water heating can account for up to a third of the energy used in the home. Compliance with the state government’s ‘5 STAR PLUS’ standards which are designed to meet the minimum 5 star energy efficiency standards of the BCA is a mandatory requirement. This can be achieved through the use of a solar or 5 star rated gas hot water system or a high

energy efficient electric heat pump.

energy rating of 4 stars.

•

Insulating the roof will greatly reduce the amount of

heat entering the home.

•

The air conditioning system should permit separate

control of the living and sleeping areas.

A feature of the local microclimate is the potential for brisk morning easterly and afternoon sea breezes. As well as embracing Perth’s temperate climate, building design should provide wind protected areas which enable the enjoyment

of outdoor living normally associated with this climate.

•

Insulating the ceiling will ensure heating and cooling effects are maximised within living areas by reducing

the extent of energy lost into the ceiling space.

•

Heating and cooling can equate to as much as a

quarter of household energy costs and by incorporating good passive solar design, the need to heat and cool

Guiding Text:

•

Insulation is a requirement of the BCA. The R Value of insulation relates to its capacity to resist heat transfer. The higher the R Value the greater the insulating effect

a material will have.

Guiding text:

the home can be greatly reduced.

•

The hot water system should be sized according to

the number of people in the home.

•

An outdoor living area is not intended to be fully walled

Guiding Text:

and roofed but may be partially covered to the extent

permitted by the R Codes.

•

A solar system has a gas booster to ensure hot water delivery in cloudy and winter conditions. Boosting is

rarely required in summer.

•

‘Reverse Brick Veneer’ refers to the construction of

•

When considering the installation of air conditioning,

compare the effectiveness of a split system to that of a ducted refrigerated system. A split system can be

external walls where the internal leaf of brickwork is complemented by an external stud framed wall rather than traditional ‘double brick’ construction. For areas with a direct solar load this form of construction can contribute to energy efficiency. Insulated wall framing clad in a reflective material can provide more satisfactory insulating characteristics than double brick walls which tend to store heat during hot days and radiate it into the home at night. Reverse brick veneer can also have the added advantage of introducing an element of variety to the elevations through the use of

an additional wall material.

•

An outdoor living area is to be directly accessible from

an internal living area and should be located to best

•

The hot water unit should be located as closely as possible to the area of most use – i.e. Kitchen

or Bathroom.

significantly cheaper to run.

facilitate winter solar penetration.

•

Ensure the thermostat is programmable to ensure

an effective response to seasonal climate conditions.

•

An outdoor living area should be planned such that wind protection is provided by the mass of the home or neighbouring home in conjunction with the use of

screen walls or buffer planting at the side of the home.

•

When deciding on what system to install consider the noise implications for neighbours as described

in clause 4.6.3.

ENERGY RATING

INDICATIVE OUTDOOR LIVING AREAS

4.2.3 THE REFRIGERATOR LOCATION IS COOL AND

WELL VENTILATED

•

The insulating characteristics of double brick external walls can be greatly improved by the use of foam

insulation within the cavity space.

The refrigerator is a major user of household energy.

Guiding Text:

•

To ensure maximum energy efficiency the refrigerator should not be located next to an

oven or in direct sunlight.

•

Ensure adequate ventilation at the rear, top and

sides of the refrigerator.

•

Select a refrigerator with a high energy efficiency

rating (above 3.5 stars.)

11 | The Village at Wellard Residential Design Guidelines

The Village at Wellard Residential Design Guidelines | 12

www.energyrating.gov.au

Guiding Text:

4.2.4 ENERGY EFFICIENT LIGHTING HAS BEEN

INSTALLED THROUGHOUT THE HOME

4.4 GARDEN DESIGN

4.5 WASTE AND RECYCLING

•

The majority of plants are natives or drought resistant.

4.4.1 A MINIMAL WATER USE GARDEN HAS

BEEN INSTALLED.

4.5.1 CONSTRUCTION WASTE

Guiding Text:

•

The extent of lawn has been minimised.

Builders are encouraged to take steps to minimise the production of non-recyclable waste and the impact of their construction activities on neighbouring properties.

A construction management plan should be developed to

minimise the amount of waste and to facilitate recycling.

•

Installing compact fluorescent light fittings in lieu of incandescent and halogen downlights can equate to

annual savings in the hundreds of dollars.

Gardens installed by the buyer should be designed to

minimise water use as well as contributing to the passive environmental aspects of the home.

- The lawn species is low water tolerant.

•

electronically controlled irrigation system with a rain sensor and efficient water delivery through pop up sprays for lawns and drippers and coarse drop sprays for gardens has been installed. Subterranean water delivery should be considered for garden areas

utilising scheme water.

•

Halogen lamps generate a large amount of heat and generally don’t have a long lifespan like

fluorescent lamps.

INDICATIVE LANDSCAPING

Guiding Text:

A strategy should be developed to minimise the amount of

waste required to be disposed of through landfill.

•

Low voltage transformers for down lights are not

necessarily energy efficient.

•

75mm of mulch has been applied over non lawn

garden areas.

Initiatives to be considered include the following:

4.3

WATER USE

•

Set in place designated waste storage areas to enable an extent of on-site sorting. i.e. separate bins/areas for metal, timber, plasterboard, masonry, glass/ceramics

etc, as opposed to a single general waste pile;

•

Paved areas are planned to direct rainwater run off

onto garden areas.

In keeping with the principles of sustainable development, dwellings should be designed to minimise water consumption. All plumbing fittings shall be water efficient and comply with the requirements of the Water Efficiency

Labelling Scheme (WELS) star rating system.

•

A soil conditioner is used to improve the soil in

garden areas.

•

Educate subcontractors to dispose of waste accordingly, thus facilitating opportunities for sorting

and recycling;

•

The strategic placing of trees and shrubs around the

home can ameliorate the effects of intense summer

4.3.1 FITTINGS AND FIXTURES

sun and provide a buffer to strong breezes.

•

Utilise waste collection agencies who carry out

further sorting/recycling off-site;

•

Each toilet shall have a 4.5/3 litre (minimum 4 star

rated) dual flush cistern.

•

Trees and shrubs can be used to assist in shading

on the east and west sides of the home

•

Regularly remove waste from the site to minimise unsightly mess and the potential for spillage into neighbouring properties. An orderly building operation should be promoted thus reinforcing for subcontractors

the message that they should take some responsibility

•

All shower heads shall be a minimum 3 star rated.

•

On the north side deciduous trees and climbers on pergolas can reduce the impact of summer sun whilst

allowing winter sunlight to penetrate the home.

•

All tapware is to be minimum 4 star rated.

4.3.2 GREYWATER RECYCLING SYSTEM

•

Shade paved areas around the home to minimise the

extent of summer heat build up.

for the minimisation of waste;

- Consider the use of grey water recycling for use on garden areas as a way to reduce the demand

on scheme water.

•

Maximise the use of pre-fabricated components in

order to achieve reductions in the extent of waste,

•

Installing lawn or planting rather than paving in front  of unprotected north facing openings will reduce heat

load on the home.

i.e. the use of steel or timber roof trusses;

•

Accurately order amounts and lengths of materials

thus avoiding excessive waste;

4.3.3 WATER TANKS

- Consider the use of rain water tanks to collect and

store water for such uses as garden watering or toilet flushing.

•

Store and recycle useable brick cut-offs into rendered

walls rather then sending to landfill;

Information on Waterwise garden design including the selection  of  plant  species  for a variety of  garden  styles  is  available  on the Water Corporation website, www.watercorporation.com.au.

•

Store timber off-cuts on site for mulching and use for

garden areas;

•

Maximise the use of non-hazardous materials, i.e. low emission water based sealers and paints rather than

epoxy/oil based;

13 | The Village at Wellard Residential Design Guidelines

The Village at Wellard Residential Design Guidelines | 14

•

Use certified plantation timber only. i.e. plantation pine trusses and laminated veneer timber beams rather

than hardwood rafters and structural beams;

•

Utilise the minimum front setbacks to provide windows facing onto the street and thus maintain

an easily surveyed front garden;

4.6 COMMUNITY & SOCIAL CONSIDERATIONS

MAINTAINING VISIBILITY

4.6.1 IMPACT ON NEIGHBOURS

Construction shall comply with the Town of Kwinana hours of work and also have due respect for neighbours by not playing loud radios and refraining from the use of loud or

objectionable language on the site.

•

Front gardens and rear outdoor areas at laneways should be well lit; possibly incorporating motion

activated light fittings;

•

Provide secure storage areas, fencing and the use of security agents in the final stages of construction in order to minimise opportunities for break in and theft; a major source of additional cost for the residential

construction industry.

•

Provide secure connections to rear laneways through

the installation of lockable garage doors and gateways;

4.6.2 PUBLIC SAFETY AND AMENITY

•

Upgrade sliding door and window security through the application of deadlocks and consider stainless steel

security mesh insect screens.

The design of homes should assist in the creation of a safe

and enjoyable residential environment.

4.5.2 SITE MANAGEMENT

Guiding Text:

Guiding Text:

- The potential for intrusion by dust or rubbish from the site into neighbouring properties should be minimised by watering down or the use of screening such as

hessian or shade cloth clad fencing.

•

Maintain visibility over streets, laneways and public open spaces from surrounding buildings by providing ample windows from habitable rooms facing or overlooking the public domain. A minimum of one major opening to a habitable room shall overlook any primary or secondary street. For 2 storey homes on cottage lots where a habitable room is constructed above the garage, window openings or balconies should provide passive surveillance opportunities

over the rear laneway.

- Stormwater is to be retained on site in order to reduce the extent of water born contaminants being

deposited into waterways.

Balcony and door openings to provide outlook over laneway

4.5.3 HOUSEHOLD WASTE

Waste minimisation within the household is encouraged

particularly the source separation and storage of

recyclable materials.

•

Design front and dividing fences forward of the building

line to be low in height in order to promote visibility;

Community amenity and health should  be  enhanced by the appropriate collection, storage and treatment of

household waste.

•

Design and maintain landscaping to minimise visual

obstruction. i.e. utilising planting such as low hedges and tree species with a high canopy;

Guiding Text:

•

Bin storage and external garden waste storage areas should be sited to reduce their visual impact and

designed to compliment the architecture and environs.

•

Appropriate space should be provided within the dwelling to facilitate the separation of recyclable waste from non recyclable. This can be achieved through the provision in the kitchen area of 1 bin for recyclable

waste and 1 bin for non recyclable waste.

15 | The Village at Wellard Residential Design Guidelines

The Village at Wellard Residential Design Guidelines | 16

4.6.3 ATTENUATION OF NOISE

4.6.4 ROBUST DESIGN

Adaptable Housing

4.7 “QUIET HOUSE” DESIGN

Due to the smaller lot sizes common in present day subdivisions, air conditioning equipment locations should be carefully considered in order to safeguard occupants and neighbours against loss of amenity caused by undue sound

being transmitted between dwellings.

Robust design principles embrace an approach to building design and construction which is safe; meets the needs of people across a range of abilities and ages;

and is adaptable to the changing needs of users.

This refers to a flexibility of home design which will

accommodate the varied and changing needs of the user with a minimum of disruption and cost.

Buyers are advised that depending on location the noise levels emanating from the railway may be noticeable. While strict noise and vibration criteria are set by the Department of Environment it is recommended that the application of “Quiet House” design principles be considered for homes

close to the railway. These include:

Elements to be considered include:

Guiding text:

•

Home Based Business: Consider the provision of a flexible space which can be used as a home office if required. i.e. At ground floor level; or in a roof space; or over a garage. Make provision for access to technology through sufficient internal cabling to

appropriate locations suitable for a home office.

Non compliance with noise regulations can result in

significant penalties for both the homeowner and the installer of noise producing equipment.

Universal Accessibility

•

Locating bedrooms away from the potential

noise source;

Universal Accessibility refers to the concept that housing design should cater for all users irrespective of age or mobility. i.e. catering for an aging population. This is best done at the design stage with elements to be considered

including the following:

Guiding Text:

•

Locating non-noise sensitive rooms closest to the

potential noise source;

•

Noise generating features such as air conditioning or evaporative cooling plant are to be appropriately located or otherwise acoustically screened to within the limits set out in the Environmental Protection

(Noise) Regulations 1997.

•

Consider the design’s ability to accommodate universal access elements at a later stage (if not

provided initially.)

•

Upgrade external doors to solid core with door seals;

•

Parking designed with suitable paving gradients and

additional clear space;

- Sealing of eaves;

•

The provision of roof and ceiling insulation;

•

Liveability: the home design should be sufficiently flexible to allow for a variety of furniture layouts and

occupancy types such as family or rental.

•

Accessible paths and ramps;

•

Acoustic planning considerations include:

•

External walls featuring double brick construction;

•

Reduced threshold heights at doorways;

–

Select a quiet model rather than the cheapest available;

Ask the supplier/installer to guarantee the suitability of the model selected;

Locate the equipment at ground level with a solid barrier such as a wall or fence to moderate the impact on neighbours;

Avoid reflected noise issues by locating equipment away from walls facing a neighbouring property;

Install equipment away from an existing neighbour’s outdoor living area or door/window openings;

Install equipment in a service area toward the rear of the property or on a rear wall well clear of side boundaries;

Install equipment on the rear of a roof so that it is not visible from any public area and the main noise impact is toward the rear of the property.

•

For glazing facing the potential noise source; the use of thicker than normal or laminated glass with opening windows recommended to feature casement sashes

in timber or commercial steel and with compressible

•

Increased widths at doorways, passages and stairs;

–

- The type of door handle used;

Additional information on accessibility and adaptability may be obtained from Australian Standard 4299 -1995 ‘Adaptable Housing’, Australian Standard 1428 -1998 ‘Design for Access and Mobility’ and The Master

Builders Association publication ‘Housing for Life’.

•

Slip resistant floor finishes;

acoustic seals.

–

•

Lower window sill heights to enable viewing from

the seated position;

–

•

Power point, light switch and telephone point

locations and heights;

The Healthy Home

–

- Adequate lighting levels;

When designing and building a home, consideration should be given to making it as safe and healthy for users as possible. The impact of pollutants or other dangers can be

minimised by:

–

•

Kitchen and bathroom cabinet design; including reduced bench heights and open spaces under sinks

and basins for wheelchair access;

–

•

Toilet and bathroom accessibility and the potential

to add grab rails;

•

Using materials that are non-toxic and do not reduce indoor air quality i.e. Treat timbers with organic oil or mild water based finishes rather than preservatives and coatings containing materials such as formaldehyde and

heavy metals or which result in VOCs;

•

For 2 storey homes consider a straight stair which will

enable the possible future installation of a chair lift.

•

Compliance with noise regulations is a statutory

requirement which is administered by the Town of Kwinana.

•

Select materials that are easily cleaned, durable and

require minimal maintenance.

•

Avoid the use of wood or kerosene heaters and where

gas is used, ensure the level of ventilation is sufficient;

•

Use less toxic and more environmentally friendly disposable products (such as cleaning products) within

the home;

•

Design cabinets with child proof latches;

•

Utilise slip resistant floor finishes.

17 | The Village at Wellard Residential Design Guidelines

The Village at Wellard Residential Design Guidelines | 18

5

Common Conditions

5.1 BUILDING FORM

INDICATIVE BUILDING FORM

5.1.1 BUILDING HEIGHT

The maximum allowable height is 2 storeys, with a third habitable level permitted within the roof space and additional vertical emphasis permitted at

nominated landmark locations.

5.1.2 ROOF FORM

In order to promote a consistency of residential

development, roof pitches shall be at 24.3° minimum and 42° maximum.

Areas of flat roof are to be hidden behind parapets except

where expressed as awnings.

Low mono-pitched skillion roofs will be acceptable only when the design is considered to be of sufficient merit

and otherwise in the spirit of the guidelines.

5.1.3 MINIMUM EAVES OVERHANG

INDICATIVE OF ARTICULATED ELEVATIONS

In line with sustainability principles, the impact if direct sunlight on walls and openings shall be mitigated by a minimum eaves overhang of 450mm. This shall apply to all eaves, except where limited by side setback requirements,   or at areas of extended roof cover, such as verandahs, entry portico’s and awnings, or at non-habitable areas

such as garages, stores, robes and fireplaces.

5.1.4 SECONDARY STREET ELEVATIONS

To promote security and provide visual interest, developments on corner lots are required to address both the primary and secondary streets. The secondary street elevation is to be articulated and feature a suitable level of detail including windows which are consistent with that of

the primary street elevation.

In line with the requirement for corner  residences  to address both streets, any fencing along a corner truncation and at least the first 1/3rd of the secondary street boundary shall be a continuation of low or visually permeable fencing

as required for front fencing.

The balance of secondary street fencing may be up to 1.8m high to suit particular privacy requirements, with any  services such as hot water units to be located behind this

and screened from public areas.

19 | The Village at Wellard Residential Design Guidelines

The Village at Wellard Residential Design Guidelines | 20

5.1.5 ARTICULATION OF FACADES

5.1.8 GEOTECHNICAL CONDITIONS

INDICATIVE OF LANDMARK ELEMENTS

METAL ROOFING

Elevations to streets and Public Open Spaces are to be articulated to feature clearly defined architectural elements,

including:

It is the responsibility of the property purchaser to ensure that the structural design of buildings and associated structures including boundary and garden walls is suitable

for the site conditions applicable to the lot.

–

Defined entries with expressed roof forms, portico’s, glazing, etc;

The avoidance of blank façades through the provision of projections and indentations in the

5.1.9 ROLLER SHUTTERS

–

Roller shutters shall not be installed to any window or

floor plan with resultant shadow effects and corresponding roof elements;

The application of awnings and other shading devices;

Façades broken by balcony projections on 2 storey development; and

Accent materials and colours applied to specific elements of the built form.

doorway visible from a public space, apart from garage

door openings.

TILE ROOFING

–

–

5.1.10 GARAGE MATERIALS

At minimum, a garage for two cars shall be provided and be

integrated with the dwelling and match the style, material and colours.

–

5.2 BUILDING MATERIALS

5.1.6 LANDMARK LOCATIONS

WALL MATERIALS

5.2.1 ROOF MATERIALS

Key locations with a high visibility such as corner lots, lots at the end of a street vista, or adjacent to a public open space are identified on the Detailed Area Plans as

“landmark” sites.

Recommended roofing materials are as follows:

–

Corrugated sheet metal in Colorbond or zincalume finish. Colours are preferred to be of neutral or low visual impact. Bright or dark, heat absorbing colours such as Colorbond “Cottage Green”, “Headland”, “Blue Ridge”, “Manor Red”, “Deep Ocean” or “Night Sky” are not recommended;

Slate or slate-style roofing tiles; and

Clay or concrete roofing tiles. Overly bright colours such as reds, greens or blues are not recommended.

In order to facilitate mental recognition of locations within a neighbourhood it is recommended that landmark locations be treated with additional architectural emphasis such as additional building height, distinctive roof forms, articulation of wall elements, or the bold use of materials, colour, detailing, etc. Residences constructed on landmark sites should draw attention to the location whilst reinforcing the

sense of architecture identity.

–

–

5.2.2 WALL MATERIALS

5.1.7 SITE LEVELS AND RETAINING WALLS

Walls shall feature a composite of construction materials. A dominant masonry material is to be complemented by minor elements of alternative finishes or materials with

selections from the following:

In order to accommodate a residence additional siteworks

and retaining walls may be required.

Additional retaining on existing retained boundaries is not permitted. Existing site levels may not be raised by more than 200mm without the specific approval of the

Town of Kwinana.

–

–

–

–

–

Face and rendered/painted brickwork; Stone cladding;

Clear glazing;

Weatherboards, painted or natural finish;

Corrugated sheet metal cladding in custom or mini orb profile in Colorbond or zincalume finish; and

Compressed fibre cement cladding.

For buildings proposed to be constructed adjacent to or on top of existing lot retaining walls, reference should firstly be made to the applicable technical note available

from the Seller.

–

Tilt-up or pre-cast concrete will be only approved for internal

walls or where the design exhibits sufficient components of detail and glazing to satisfy the spirit of the guidelines.

21 | The Village at Wellard Residential Design Guidelines

The Village at Wellard Residential Design Guidelines | 22

5.3 STREET PRESENTATION

– The house shall be designed so as to maximise its presentation to the street. Rooms shall run across  the lot with side setbacks to be as small as possible. Designs which feature narrow building  frontages with large side setbacks are not permitted.

–  Provision is to be made for the storage of rubbish  bins in such a way as they are screened from public

view and can be easily accessed for collection.

5.5.4 LANEWAY RETAINING

5.5.8 UNIFORM FENCING BY DEVELOPER

All laneway lots shall be finished at the laneway with appropriate limestone or masonry retaining which protects the edge integrity of the lane-roadway, and provides

a barrier between the laneway and any rear fencing.

At the developer’s discretion, uniform fencing may be provided to boundaries of lots abutting selected roads and Public Open Space, which may differ from the fencing

requirements in these guidelines.

5.5 FENCING GENERALLY

– See sections 6.0 – 10.0 for fencing requirements relating to specific housing typologies

Where uniform fencing is provided by the developer, it may not be removed, altered, marked, damaged or allowed to

fall into a state of disrepair by the lot owner.

5.5.5 FRONT FENCES GENERALLY

Return Fences

5.4 SERVICES

In order to enhance the sense of community fostered by the Village at Wellard, any fencing installed by buyers to

front boundaries is required to be low and open.

–

Parkland and cottage lots may be provided with service easements for the connection of water, gas and electrical services. All meters are to be contained within the  easement  with  screening or other architectural treatments to be integrated into the landscape or building design.

Waste and vent pipes, cable ducts, air-conditioning and evaporative cooling plant, television antennae, satellite dishes, hot water storage tanks and clothes drying areas are to be concealed from street or public view. Solar panels may be visible where they are in the same plane as the roof and there is no alternative location, which affords a suitable level

of solar efficiency.

Where air-conditioning or evaporative cooling plant is roof mounted it is to be finished in a colour to match that of the roofing material.

All air-conditioning and evaporative cooling plant shall be located and  acoustically  screened  so  as to minimise the level of noise intrusion into neighbouring properties to within limits set out by

the Environment Protection Act 1986: Environmental

Protection (Noise Regulations 1997).

Infill return fences from a side or secondary street boundary to the building shall be in a material which

matches the building or the side fencing.

5.5.9 CORNER LOTS

To  maintain an open streetscape that facilitates visibility  and cross surveillance, this shall be in the form of rendered masonry or limestone piers to a maximum height of 1.2m and infilled with rendered masonry, brick, limestone or

visually permeable panels to a maximum height of 1.0m.

On the secondary street frontage, the fencing is to be    a minimum permissible standard of a proprietary brand

capped metal fence to a maximum height of 1.8m, with   the top 0.3m recommended to be a component of visually

permeable panels.

Where a building such as a garage or carport is located less than 0.8m from the side boundary, the return fence must

be in a material that matches that of the building.

–

5.5.1 FRONT FENCES PROVIDED BY THE DEVELOPER

This fence is not permitted to extend forward of a point 4m back from the front building line unless it is low or visually

permeable as is required for a front fence.

5.5.6 DIVIDING FENCES

Where front walls or fences are provided by the developer,

these shall not be added to through the installation of additional solid or open fencing material.

The minimum permissible standard for fences dividing adjoining properties is 1.8m high proprietary brand capped Colorbond metal fencing which is not to project past the

adjacent building line.

5.6 EXISTING MATURE TREES

5.5.2 RETURN FENCING AT REAR LANEWAYS

In designing the Village at Wellard considerable effort has

been devoted to retaining as many existing mature trees as possible.

–

Where a home is not constructed hard up to a nominated zero lot line, resulting in a gap between the building and the side boundary fence, this shall be closed off by return

fencing which matches the side fence in material and colour.

A dividing fence in front of the building line is to be in

material as required for a front fence.

Where an existing mature tree occurs within a street verge or lot, it is not permitted to be removed without permission from the Town of Kwinana. A residence proposed for an affected lot is to be designed so that the driveway

crossover is located clear of any existing tree.

–

5.5.7 DIVIDING FENCES FORWARD OF THE

BUILDING LINE

5.5.3 RETAINING WALLS CONSTRUCTED

BY THE BUYER

Dividing fences forward of the building line shall be as for

the front fence with piers to a maximum height of 1.2m and infill panels to a maximum of 1.0m.

All retaining walls constructed by the buyer which are visible from any public place shall match, in material and finish, retaining walls provided throughout the estate by

the developer.

Materials shall be as for those of a front fence.

23 | The Village at Wellard Residential Design Guidelines

The Village at Wellard Residential Design Guidelines | 24

6

Parkland Lots

6.2 OPEN SPACE

6.4 STREET OUTLOOK

6.7 DUPLEX LOTS

A minimum of 30% of the lot area is required to be

provided as open space.

Dwellings designed for Parkland Lots will potentially feature living areas located toward the rear of the property taking advantage of the park outlook and resulting in a reduced

emphasis on streetfront security.

Lots designated as suitable for duplex development are

indicated on the Detailed Area Plan.

6.3 OUTDOOR LIVING AREA

6.7.1 FORM OF BUILDING

An outdoor living area with a minimum area of 30sqm, and  a minimum dimension of 4m, which is directly accessed from a living area, is to be provided in a location to best

facilitate winter solar penetration.

To minimise this, a habitable room with windows affording the opportunity of passive street front surveillance should be provided at the front of the dwelling. For two-storey dwellings it is recommended that a balcony be provided

to afford greater surveillance opportunities as well as

enhancing the street elevation.

In order to create an appropriate vertical emphasis, each dwelling contained in a duplex development is

recommended to be two storeys high.

Each dwelling is to have a minimum open space of 30%

of the apportioned lot area and a minimum outdoor living area of 20sqm with a minimum dimension of 4m.

For lots which have predominantly east-west orientation, any two-storey development is to be located to the front 65% of the lot to minimise any potential overshadowing

to neighbouring outdoor living areas.

6.5 STORES & OUTBUILDINGS

6.7.2 PARKING

A store with a minimum area of 4sqm is to be provided under the main roof of the residence and may be accessed

either from the exterior or within the garage area.

Garages may be located off the primary and

secondary streets.

Any outbuilding not constructed of the same materials

as the main residence is not permitted to be visible from  a street, park or other public space.

INDICATIVE OF FENCING

All outbuildings with a floor area in excess of 10sqm or

a height of 2m or more must be constructed of the same roof and wall materials as the main residence.

6.6 FENCES

See clause 5.5 for general fencing requirements.

At the abutment with developer provided rear (POS) boundary fencing, 1.8m high dividing fencing is to be raked down to a maximum height of 1m over a minimum

horizontal distance of 1m.

25 | The Village at Wellard Residential Design Guidelines

The Village at Wellard Residential Design Guidelines | 26

6.1 MINIMUM SETBACKS

Min

Max

Front

Garage Dwelling

As per R Codes 2m

n/a 4m

*Side

(zero lot line)

Ground Floor First Floor

nil

As per R Codes

n/a n/a

**Side

As per R Codes

n/a

Rear (POS)

As per R Codes

Secondary Street

As per R Codes

n/a

*   Refer to clause 5.1.7 for building adjacent to or on top of existing retaining walls.

** At a side boundary opposite a zero lot line boundary, the minimum setback for a

habitable room with a major opening facing the boundary is 2m in order to maximise solar access.

7

Cottage Lots

7.2 OPEN SPACE

7.4 PARKING

7.7 LETTERBOX LOCATION

A minimum of 30% of the lot area is required to be

provided as open space.

Garages are required to be located off  the  rear  laneway and are to be complete with a segmented panel lift or roller door. Vehicular access is not permitted off the primary

street frontage.

Where a cottage lot has a front retaining wall provided by the developer, the letterbox shall be of a standard design and is to be located on the primary street in a standard

position on the wall, to be advised by the developer.

7.3 OUTDOOR LIVING AREA

An outdoor living area with a minimum area of 30sqm, a minimum width of 4m and directly accessed from a living area, is to be provided in a position to best facilitate winter

solar penetration.

7.5 STORES & OUTBUILDINGS

7.8 DUPLEX LOTS

A store with a minimum area of 4sqm is to be provided

under the main roof of the residence and may be accessed either from the exterior or within the garage area.

Lots designated as suitable for duplex development are

indicated on the Detailed Area Plan.

7.8.1 FORM OF BUILDING

Any outbuilding not constructed of the same materials as the main residence is not permitted to be visible from

a street or other public area.

In order to create an appropriate vertical emphasis each dwelling contained in a duplex development is

recommended to be two storeys high.

All outbuildings with a floor area in excess of 10sqm or

a height of 2m or more must be constructed of the same roof and wall materials as the main residence.

Each dwelling is to have a minimum open space of 30%

of the apportioned lot area and a minimum outdoor living area of 20sqm with a minimum dimension of 4m.

7.6 FENCES

7.8.2 PARKING

See clause 5.5 for general fencing requirements.

Garages are required to be located off  the  rear  laneway and are to be complete with a segmented panel lift or roller door. Vehicular access is not permitted off the primary

street frontage.

Laneway Fences

Fencing to a laneway shall be either solid rendered masonry or proprietary brand, capped, colorbond metal fencing including a component of visually permeable panels as per the applicable covenants. Solid fencing shall be limited to

storage/service areas.

INDICATIVE OF FENCING

27 | The Village at Wellard Residential Design Guidelines

The Village at Wellard Residential Design Guidelines | 28

7.1 MINIMUM SETBACKS

Min

Max

Front

2m

4m

*Side

(zero lot line)

Ground Floor First Floor

nil

As per R Codes

n/a n/a

**Side

As per R Codes

n/a

Rear (Laneway)

Garage Ground Floor First Floor

0.5m

Residence 1.5m Nil

n/a n/a n/a

Secondary Street

As per R Codes

n/a

* Refer to clause 5.1.7 for building adjacent to or on top of existing retaining walls

** At a side boundary opposite a zero lot line boundary the minimum setback for a habitable room with a major opening facing the boundary is 2m in order to maximise solar access.

8	Traditional Lots

8.2 OPEN SPACE

8.4 STORES & OUTBUILDINGS

Wall Materials:

Masonry walls are  recommended  however  fibre cement and weatherboards are acceptable although vinyl weatherboards and rough timber can cause problems in a fire through warping and catching sparks.

A minimum of 45% of the lot area is required to be

provided as open space.

Any outbuilding with a floor area in excess of 10sqm or

2m or more in height must be constructed of the same roof and wall materials as the main residence.

8.3 OUTDOOR LIVING AREA

Any outbuilding not constructed of the same materials

as the main residence is not permitted to be visible from  a street or other public area.

Decks:

If considering an elevated deck area be aware that rough sawn timber catches dust which is highly flammable.

Exposed timber joists should be a dense hardwood like jarrah and be given a smooth or painted finish. Flammable

coatings such as resinous compounds should not be used.

An outdoor living area with a minimum area of 30sqm, a minimum dimension of 4m and directly accessed from a living area is to be provided in a location to best facilitate

winter solar penetration.

8.5 FENCES

See clause 5.5 for general fencing requirements.

Gardens:

Fuel such as tree branches, long dry grass, dead leaves and twigs and flammable shrubs should be kept well back from the house.

8.6 BUSHFIRE DESIGN CONSIDERATIONS

Due to the proximity of some lots to the Bush Forever Reserve or the Leda Nature Reserve, affected buyers should refer to the Fire and Emergency Services Authority of Western Australia, Homeowners Bush Fire Survival

Manual current at the time when planning their homes.

Relevant design considerations include but are not

limited to:

Roofing Materials:

Metal roofing is generally preferable to tiles as it is less likely to permit sparks to enter through the roof, tiled roofs should include fireproof sarking between the roofing and timber roof framing.

Skylights:

Should use flat wire meshed glass and have a removable external cover for fire protection.

Roof Mounted Air Conditioners:

Should have a suitable metal screen to prevent the entry of sparks.

Floors:

Concrete slab or ground is recommended however where a raised floor on stilts is employed this should be located as close as possible to ground level and the under floor space should be enclosed.

29 | The Village at Wellard Residential Design Guidelines

The Village at Wellard Residential Design Guidelines | 30

8.1 MINIMUM SETBACKS

Min

Max

Front

*Garage Dwelling

As per R Codes 3m

n/a 5m

Side

As per R Codes

Rear

As per R Codes

Secondary Street

As per R Codes

n/a

* Note Where the garage is configured so that vehicles are parked parallel to the street alignment the applicable front setback may be reduced to 2m.

9	Townhouse Lots

9.1 BUILDING FORM

9.3 SITE LEVELS AND RETAINING WALLS

9.8 FENCES

INDICATIVE OF FENCING

Development is to take into account the location of the

lots near the village centre commercial precinct and in order to provide an appropriate vertical urban edge shall take the form of two storey townhouses fronting onto  Lambeth Circle.  A third habitable level is permitted within the roof space.

Additional retaining on existing retained boundaries is not permitted. Existing site levels may not be raised by more than 200mm without specific approval from the

Town of Kwinana.

See clause 5.5 for general fencing requirements.

Where uniform fencing is provided by the developer, it may

not be removed, altered, marked, damaged or allowed to fall into a state of disrepair by the lot owner.

For buildings proposed to be constructed adjacent to or

on top of existing lot retaining walls, reference should firstly be made to the applicable technical note available from

the Seller.

Laneway Fences

Development is preferred to be built hard up to both side

boundaries (nil setback) over two storeys resulting in attached townhouses.

See clause 5.5 for general fencing requirements.

Fencing to a laneway shall be either solid rendered masonry or proprietary brand, capped, colorbond metal fencing including a component of visually permeable panels as

per the applicable covenants. Solid fencing shall be limited

to storage/service areas.

9.4 OPEN SPACE

A minimum of 30% of the lot area is required to be

provided as open space.

9.5 OUTDOOR LIVING AREA

9.9 LETTERBOX LOCATION

An outdoor living area with a minimum area of 20sqm, a minimum width of 4m and directly accessed from a living area, is to be provided in a position to best facilitate winter

solar penetration.

Where a townhouse lot has a front retaining wall provided  by the developer, the letterbox shall be of a standard design and is to be located on the primary street in a standard

position on the wall, to be advised by the developer.

9.6 PARKING

9.10 DUPLEX LOTS

Garages are required to be located off the rear laneway and are to be complete with a segmented panel lift or roller  door. Vehicular access is not permitted off the primary

street frontage.

Lots which are designated as suitable for duplex

development, are indicated on the Detailed Area Plans.

9.10.1 FORM OF BUILDING

In order to create an appropriate vertical emphasis each dwelling contained in a duplex development is required

to be 2 storeys high.

9.7 STORES & OUTBUILDINGS

A store with a minimum area of 4sqm is to be provided under the main roof of the residence or garage and may be

accessed either from the exterior or within the garage area.

Each dwelling is to have a minimum open space of 30%    of the apportioned lot area and a minimum outdoor living

area of 20sqm with a minimum dimension of 4m.

Any outbuilding with a floor area in excess of 10sqm or

2m or more in height must be constructed of the same roof and wall materials as the main residence.

9.10.2 PARKING

Any outbuilding not constructed of the same materials as the main residence is not permitted to be visible from

a street or other public area.

Garages are required to be located off the rear laneway and are to be complete with a segmented panel lift or  roller door. Vehicular access is not permitted off a primary

street frontage.

31 | The Village at Wellard Residential Design Guidelines

The Village at Wellard Residential Design Guidelines | 32

9.2 MINIMUM SETBACKS

Min

Max

Front

2m

4m

*Side

nil

n/a

**Side

As per R Codes

n/a

Rear (Laneway)

Garage Ground Floor First Floor

0.5m

Residence 1.5m Nil

Secondary Street

As per R Codes

10

Homestead Lots

10.1 MINIMUM SETBACK

10.4 FENCES

10.5 BUSHFIRE DESIGN CONSIDERATIONS

Gardens:

Fuel such as tree branches, long dry grass, dead leaves and twigs and flammable shrubs should be kept well back from the house.

Due to the larger size of Homestead Lots, a building envelope is defined in order to create a buffer between homes and to facilitate a tree preservation area. Generally the building envelope will be between 600sqm to 750sqm  in area and will be set back a minimum of 5m from a front

boundary and 4m from a side boundary.

See clause 5.5 for general fencing requirements.

Due to the proximity of the lots to existing Special Residential Development featuring an extent of retained bush, buyers should refer to the Fire and Emergency Services Authority of Western Australia, Homeowners Bush Fire Survival Manual current at the time when

planning their homes.

Front Fences

Fencing in front of the building envelope shall be as per

Clause 5.5, and in any event shall comply with the requirements of the R Codes.

The allowable building envelope is defined on the applicable DAP and all buildings are required to be contained within

this area.

Other Boundary Fences

Relevant design considerations include but are not

limited to:

All other boundary fencing shall be of a rural and open nature with materials such as fibre cement, pickets,

sheet iron etc. prohibited by the Town of Kwinana.

Roofing Materials:

Metal roofing is generally preferable to tiles as it is less likely to permit sparks to enter through the roof, tiled roofs should include fireproof sarking between the roofing and timber roof framing.

10.2 TREE PRESERVATION AREA

Fences at or within the Building Envelope

The area outside the building envelope and behind the 5m front setback is designated as the  tree  preservation  area. No development including driveways, sheds, buildings or the storage of materials is permitted within the tree preservation area. The buyer should refer to Town Planning Scheme

Policy for restrictions on the removal of Indigenous vegetation.

Fences at or within the building envelope may, at the discretion of the Town of Kwinana, be of a closed or screening nature provided the scale or colour of the

fence does not detract from the amenity of the area.

Skylights:

Should use flat wire meshed glass and have a removable external cover for fire protection.

Roof Mounted Air Conditioners:

Should have a suitable metal screen to prevent the entry of sparks.

10.3 CONSTRUCTION MATERIALS

–

No roofs of homes or carport/garages or outbuildings shall be constructed of highly reflective materials such as unpainted zincalume.

All dwellings and carport/garages shall be constructed of stone or masonry unless otherwise approved by the Town of Kwinana.

Outbuildings may be constructed of metal or other materials approved by the Town, provided the external colour is of a neutral and non-glaring finish.

Floors:

Concrete slab or ground is recommended however where a raised floor on stilts is employed this should be located as close as possible to ground level and the underfloor space should be enclosed.

–

–

Wall Materials:

Masonry walls are recommended however fibre cement and weatherboards are acceptable although vinyl weatherboards and rough timber can cause problems in a fire through warping and catching sparks.

Decks:

If considering an elevated deck area be aware that rough sawn timber catches dust which is highly flammable.

Exposed timber joists should be a dense hardwood like jarrah and be given a smooth or painted finish. Flammable

coatings such as resinous compounds should not be used.

33 | The Village at Wellard Residential Design Guidelines

The Village at Wellard Residential Design Guidelines | 34

Indicative of Homestead Homes

This material has been prepared with care, however it is an indicative guide only and believed to be correct at the time of printing. All information (including but not limited to prices) is subject to change without notice at Peet’s absolute discretion and cannot form part of any offer or contract. The Seller reserves the right to presell, release and withhold certain packages from sale at any time. Whilst reasonable care has been taken in preparing this material, the Seller, its representatives, employees and agents cannot be held responsible for any inaccuracies and this material should not be relied upon, as no warranty or representation is given or to be construed. Purchasers should obtain all necessary and relevant information, review all information carefully, make their own inquiries and obtain independent advice before proceeding. Areas and dimensions are approximates only and subject to survey and title registration dates are approximates only and subject to regulatory approvals and factors beyond Peet’s control.

The Village at Wellard Residential Design Guidelines | 36

35 | The Village at Wellard Residential Design Guide

Sales and Information Centre 13 Chiswick Parade, Wellard (opposite the Wellard Train Station)

Sales Centre Opening Hours Saturday – Wednesday 1 – 5pm

Phone: 9419 6222

thevillageatwellard.com.au

<

<

Perth 28mins

Sales and Information Centre

<

Rockingham	Wellard

4mins	Train

Station

THE VILLAGE AT WELLARD

Mandurah 22mins